WRITTEN REPORT

Project #1: Organizing and Summarizing Air Quality Data

Nebraska AirPurple Monitoring Network — February 2024 to March 2025

Submitted to: UNMC Water, Climate and Health Group

Prepared by: Aminasahra Warsame

Date: 5/3/2026

# 1. Introduction and Background

Air quality is a critical public health concern, with well-documented links between exposure to fine particulate matter (PM 2.5), coarse particulate matter (PM 10.0), and volatile organic compounds (VOCs) and adverse health outcomes including respiratory disease, cardiovascular events, and exacerbation of asthma. The U.S. Environmental Protection Agency regulates concentrations of these pollutants through the National Ambient Air Quality Standards (NAAQS), which establish health-protective thresholds for each pollutant (EPA, 2023).

Nebraska presents a distinctive air quality profile shaped by its agricultural economy, semi-arid climate, flat Great Plains topography, and susceptibility to long-range transport of wildfire smoke and dust from western states. In 2024, the UNMC Water, Climate and Health Group initiated a partnership with AirPurple to deploy a network of low-cost sensors across the state to monitor these conditions continuously. Our engineering firm was contracted through a competitive RFP process to analyze the data collected from this network for the period February 2024 through March 2025.

## 1.1 Scope of Work Summary

The analysis included: (1) data loading, cleaning, and preparation; (2) descriptive statistical analysis of VOC, PM 2.5, and PM 10.0 by sensor location; (3) identification of maximum pollutant events and investigation of potential causes; (4) evaluation of temperature and humidity effects on pollutant concentrations; (5) AQI health risk event identification; and (6) a bonus analysis of the relationship between sensor altitude and observed concentrations. All analysis was performed in Python using a Jupyter Notebook environment with the Pandas, NumPy, and Matplotlib libraries.

## 1.2 Objectives

The specific objectives, as requested by the UNMC client, were:

- Identify the five Nebraska sensor locations with the highest mean and median concentrations of VOC, PM 2.5, and PM 10.0.

- Identify the dates and sensor locations where maximum pollutant values occurred and investigate potential causes.

- Assess the effect of relative humidity and temperature on air quality.

- Identify AQI health risk events for PM 2.5 and PM 10.0, document when and where they occurred, and investigate potential causes.

- (Bonus) Evaluate whether sensor altitude affects observed air quality values.

# 2. Methods

## 2.1 Data Source

The primary data source is the AirPurple monitoring network deployed across Nebraska by the UNMC group. The dataset (AirQuality_Daily_StudentVersion.csv) contains daily observations from 27 sensor locations spanning February 23, 2024 to March 24, 2025, totaling 8,301 rows. Fields include date, sensor name, humidity, pressure, temperature, VOC, PM 1.0, PM 2.5 (pm2.5_alt), PM 10.0 (pm10.0_atm), and sensor latitude, longitude, and altitude. Supplementary sources used for AQI classification and contextual interpretation include the EPA NAAQS table (EPA, 2023), the EPA AQI Technical Assistance Document (EPA, 2018), AirNow (2024), NOAA (2024), and the Nebraska Department of Environment and Energy (NDEE, 2023).

## 2.2 Data Preparation

Data were loaded into Python using Pandas. The date column was converted to datetime format, sensor name strings were stripped of whitespace, string instances of ‘NA’ were replaced with NumPy NaN, and all numeric fields were cast to float using pd.to_numeric() with errors='coerce'. Temperature and humidity categorical variables were assigned using the client-specified thresholds. All analyses were grouped by sensor name.

## 2.3 Statistical Analysis

Mean and median concentrations were computed per sensor using groupby and agg(). The top five sensors by mean and median were identified using nlargest(). Maximum values and their associated date, sensor, coordinates, and altitude were extracted using idxmax(). Environmental factor analysis used groupby on the categorical variables. AQI classification was applied row-wise using custom Python functions based on EPA breakpoints. Pearson correlation coefficients between sensor altitude and pollutant concentrations were computed using Pandas corr().

# 3. Results and Discussion

## 3.1 Top Five Sensor Locations by Pollutant Concentration

### 3.1.1 VOC

Units: ppb. Swnphd-Ogallala recorded the highest mean VOC concentration across the dataset (399.434 ppb), followed closely by FCHD-YPS (372.463 ppb), Three Rivers Public Health Department (370.216 ppb), ELVPHD Norfolk HD 4 (360.834 ppb), and Swnphd-McCook (353.942 ppb). The median rankings were similar, with Swnphd-Ogallala again ranking first (423.082 ppb). The consistency between mean and median rankings suggests stable, persistent VOC sources at these locations rather than isolated spike events. The top VOC sensors are concentrated in the southwestern and central Nebraska region, consistent with high agricultural activity including livestock operations and chemical application associated with crop production (EPA, 2023).

| Rank | Sensor (Mean) | Mean |  | Rank | Sensor (Median) | Median |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Swnphd-Ogallala | 399.434 |  | 1 | Swnphd-Ogallala | 423.082 |
| 2 | FCHD-YPS | 372.463 |  | 2 | Swnphd-McCook | 381.468 |
| 3 | Three Rivers Public Health Dept. | 370.216 |  | 3 | Three Rivers Public Health Dept. | 376.810 |
| 4 | ELVPHD Norfolk HD 4 | 360.834 |  | 4 | FCHD-YPS | 375.384 |
| 5 | Swnphd-McCook | 353.942 |  | 5 | ELVPHD Norfolk HD 4 | 368.580 |

### 3.1.2 PM 2.5

Units: µg/m³. The Southeast District Health Department in Tecumseh (#18) recorded the highest mean PM 2.5 concentration (9.179 µg/m³), followed by Ainsworth Public School #9 (8.195 µg/m³) and ELVPHD Norfolk HD 4 (7.994 µg/m³). It is worth noting that all mean PM 2.5 values across the top five sensors remain well below the EPA NAAQS 24-hour standard of 35 µg/m³ on average, though individual days exceeded this threshold as discussed in Section 3.4. Sensors in eastern and northeastern Nebraska dominate this ranking, consistent with proximity to agricultural burning activity and regional smoke transport pathways (NOAA, 2024).

| Rank | Sensor (Mean) | Mean |  | Rank | Sensor (Median) | Median |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | #18 - SE Health Dept. Tecumseh | 9.179 |  | 1 | Ainsworth Public School #9 | 6.810 |
| 2 | Ainsworth Public School #9 | 8.195 |  | 2 | #18 - SE Health Dept. Tecumseh | 5.825 |
| 3 | ELVPHD Norfolk HD 4 | 7.994 |  | 3 | ELVPHD Norfolk HD 4 | 5.438 |
| 4 | NCDHD O'Neill #11 | 6.294 |  | 4 | #16 - Richardson County Courthouse | 4.844 |
| 5 | #16 - Richardson County Courthouse | 6.108 |  | 5 | #17 - Otoe County | 4.694 |

### 3.1.3 PM 10.0

Units: µg/m³. Broken Bow recorded by far the highest mean PM 10.0 concentration (929.679 µg/m³), which is substantially above the EPA NAAQS 24-hour standard of 150 µg/m³. Richardson County Courthouse (#16) ranked second at 701.632 µg/m³. The large gap between mean (929.679) and median (43.179) at Broken Bow indicates the presence of extreme spike events driving the mean upward, rather than a consistently elevated baseline. These spikes are most likely caused by high-wind dust events, which are common across the Nebraska plains, particularly during winter and spring months when soil moisture is low (NDEE, 2023).

| Rank | Sensor (Mean) | Mean |  | Rank | Sensor (Median) | Median |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Broken Bow | 929.679 |  | 1 | Broken Bow | 43.179 |
| 2 | #16 - Richardson County Courthouse | 701.632 |  | 2 | #16 - Richardson County Courthouse | 13.306 |
| 3 | #18 - SE Health Dept. Tecumseh | 614.227 |  | 3 | ELVPHD Norfolk HD 4 | 11.475 |
| 4 | NCDHD O'Neill #11 | 166.133 |  | 4 | #18 - SE Health Dept. Tecumseh | 11.434 |
| 5 | Swnphd-McCook | 124.227 |  | 5 | ELVPHD Wisner HD 5 | 10.936 |

## 3.2 Maximum Values: Dates, Locations, and Potential Causes

The highest single recorded value for each pollutant across the full dataset was identified and is summarized in the table below.

| Pollutant | Max Value | Date | Sensor | Coordinates | Altitude |
| --- | --- | --- | --- | --- | --- |
| VOC | 1,209.932 ppb | 2024-06-24 | Swnphd-Ogallala | 41.1283, -101.7202 | 3,220 ft |
| PM 2.5 | 56.633 µg/m³ | 2024-10-09 | ELVPHD Norfolk HD 4 | 42.0331, -97.4084 | 1,522 ft |
| PM 10.0 | 3,784.683 µg/m³ | 2025-02-18 | #16 - Richardson County Courthouse | 40.0622, -95.6024 | 1,023 ft |

### 3.2.1 VOC Maximum — June 24, 2024, Swnphd-Ogallala

The highest VOC reading of 1,209.932 ppb was recorded at the Swnphd-Ogallala sensor on June 24, 2024. This date falls in late June, which corresponds to the peak of Nebraska's summer crop growing season and a period of intensive pesticide and herbicide application in the surrounding Keith County agricultural region. VOC emissions from pesticide volatilization, livestock operations, and vehicle fuel combustion from agricultural equipment are all likely contributors. The elevated altitude of this sensor (3,220 ft) may also affect local atmospheric mixing and VOC concentration dynamics (EPA, 2023).

### 3.2.2 PM 2.5 Maximum — October 9, 2024, ELVPHD Norfolk HD 4

The maximum PM 2.5 value of 56.633 µg/m³, recorded at Norfolk in northeastern Nebraska on October 9, 2024, exceeds the EPA 24-hour NAAQS standard of 35 µg/m³ and falls in the “Unhealthy” AQI category. October is the primary post-harvest agricultural burning season in Nebraska, during which crop residue burning generates significant quantities of fine particulate matter. Long-range transport of wildfire smoke from western states also commonly affects Nebraska during fall months, and these events are frequently documented in AirNow historical data and NOAA smoke forecast archives (NOAA, 2024).

### 3.2.3 PM 10.0 Maximum — February 18, 2025, Richardson County Courthouse

The maximum PM 10.0 reading of 3,784.683 µg/m³ was recorded at the Richardson County Courthouse sensor in Falls City, Nebraska on February 18, 2025 — nearly 25 times the EPA 24-hour NAAQS standard of 150 µg/m³. This extreme value is characteristic of a high-wind dust event. February in Nebraska is associated with strong frontal wind systems capable of mobilizing topsoil and agricultural dust, particularly when snow cover is absent and soils are dry and exposed. The persistence of elevated readings at this sensor across February 18–20, 2025 is consistent with a multi-day wind event rather than a brief anomaly (NDEE, 2023).

## 3.3 Effect of Humidity on Air Quality

Mean pollutant concentrations were computed for each of the three client-defined humidity categories and are summarized below.

| Humidity Category | Mean VOC (ppb) | Mean PM 2.5 (µg/m³) | Mean PM 10.0 (µg/m³) |
| --- | --- | --- | --- |
| Low (<50%) | 279.329 | 5.182 | 77.881 |
| High (50–80%) | 252.664 | 7.119 | 82.561 |
| Very High (>80%) | 19.396 | 3.022 | 6.322 |

The results show that very high humidity (>80%) is associated with dramatically lower concentrations across all three pollutants. This is most pronounced for VOC, which dropped from 279.329 ppb at low humidity to just 19.396 ppb at very high humidity. This pattern is consistent with rain scavenging and wet deposition processes that remove particulates and VOCs from the atmosphere during high-humidity conditions, as well as reduced agricultural activity (chemical application, soil disturbance) during wet weather periods. PM 2.5 was highest in the High humidity category (7.119 µg/m³), which may reflect hygroscopic growth of fine aerosol particles at moderate humidity levels before washout occurs at very high humidity (EPA, 2023). PM 10.0 followed a similar pattern, with Low and High categories showing comparable mean values and Very High showing a sharp reduction, consistent with wet suppression of dust resuspension.

## 3.4 Effect of Temperature on Air Quality

Mean pollutant concentrations were computed for each of the four client-defined temperature categories and are summarized below.

| Temperature Category | Mean VOC (ppb) | Mean PM 2.5 (µg/m³) | Mean PM 10.0 (µg/m³) |
| --- | --- | --- | --- |
| Below Freezing (<32°F) | 257.629 | 4.949 | 276.081 |
| Cool (32–50°F) | 285.435 | 4.560 | 143.074 |
| Warm (51–70°F) | 257.016 | 3.380 | 87.429 |
| Hot (>70°F) | 282.146 | 6.970 | 20.244 |

Temperature showed a notable relationship with PM 10.0, which was highest under below-freezing conditions (276.081 µg/m³), likely driven by the extreme dust events recorded at Broken Bow and Richardson County Courthouse during January and February 2025. PM 2.5 was highest in the Hot (>70°F) category (6.970 µg/m³), consistent with increased agricultural activity, wildfire smoke, and photochemical reactions that enhance secondary aerosol formation during warm summer months. VOC concentrations were relatively elevated across all temperature categories except below freezing, reflecting the broad range of VOC sources active in Nebraska across seasons. The Cool category produced the highest mean VOC (285.435 ppb), which may reflect spring planting season chemical application occurring primarily in moderate temperature conditions.

## 3.5 AQI Health Risk Events

Each PM 2.5 and PM 10.0 observation was classified according to EPA AQI breakpoint thresholds. Days meeting or exceeding the “Unhealthy for Sensitive Groups” threshold were flagged as health risk events.

PM 2.5: 25 health risk event-days were identified across the dataset. Three of these reached the “Unhealthy” category. The top events are listed below.

| Date | Sensor | PM 2.5 (µg/m³) | AQI Category |
| --- | --- | --- | --- |
| 2024-10-09 | ELVPHD Norfolk HD 4 | 56.633 | Unhealthy |
| 2024-05-14 | #18 - SE Health Dept. Tecumseh | 56.004 | Unhealthy |
| 2024-05-14 | #16 - Richardson County Courthouse | 55.542 | Unhealthy |
| 2024-07-25 | Swnphd-Ogallala | 48.738 | Unhealthy for Sensitive Groups |
| 2024-05-14 | FCHD-YPS | 46.900 | Unhealthy for Sensitive Groups |
| 2024-05-14 | PHS Fairbury JCHL 13 | 44.479 | Unhealthy for Sensitive Groups |
| 2024-07-25 | SWNPHD-Imperial | 44.433 | Unhealthy for Sensitive Groups |
| 2024-07-25 | Swnphd-Benklemen | 44.379 | Unhealthy for Sensitive Groups |
| 2024-05-14 | Three Rivers Public Health Dept. | 44.017 | Unhealthy for Sensitive Groups |
| 2024-07-25 | Broken Bow | 43.167 | Unhealthy for Sensitive Groups |

Two dates account for the majority of PM 2.5 health risk events: May 14, 2024 and July 25, 2024. The widespread occurrence of elevated PM 2.5 across multiple sensors on May 14, 2024 (affecting at least six sensors simultaneously) strongly suggests a regional event such as long-range wildfire smoke transport from western states or a widespread agricultural burning episode. May corresponds to pre-planting field preparation and early burn-off activity in Nebraska. July 25, 2024 similarly produced events at multiple sensors across western and central Nebraska, consistent with peak wildfire season smoke transport from the Rocky Mountain region (NOAA, 2024).

PM 10.0: 392 health risk event-days were identified, substantially more than for PM 2.5. The top events are listed below.

| Date | Sensor | PM 10.0 (µg/m³) | AQI Category |
| --- | --- | --- | --- |
| 2025-02-18 | #16 - Richardson County Courthouse | 3,784.683 | Hazardous |
| 2025-02-19 | #16 - Richardson County Courthouse | 3,403.436 | Hazardous |
| 2025-02-20 | #16 - Richardson County Courthouse | 3,245.613 | Hazardous |
| 2025-01-21 | #16 - Richardson County Courthouse | 3,211.047 | Hazardous |
| 2025-02-18 | Broken Bow | 3,206.780 | Hazardous |
| 2025-02-20 | Broken Bow | 3,182.551 | Hazardous |
| 2025-02-19 | Broken Bow | 3,179.656 | Hazardous |
| 2025-01-20 | Broken Bow | 3,088.054 | Hazardous |
| 2025-02-12 | #16 - Richardson County Courthouse | 3,081.851 | Hazardous |
| 2025-02-18 | Swnphd-McCook | 3,054.484 | Hazardous |

The 392 PM 10.0 health risk days represent a significant public health concern. All top events reached the “Hazardous” AQI category, with values far exceeding the NAAQS standard. Richardson County Courthouse and Broken Bow are the most severely affected sensors. The concentration of extreme events in January and February 2025 at these locations is consistent with winter wind storms on the Great Plains, where dry, exposed agricultural soils are highly susceptible to wind erosion. These events are especially dangerous for sensitive populations including children, elderly individuals, and those with asthma, COPD, or cardiovascular conditions (EPA, 2018).

## 3.6 Bonus: Effect of Sensor Altitude on Air Quality

Pearson correlation coefficients were calculated between sensor altitude and pollutant concentrations to assess whether elevation plays a role in observed air quality values.

| Pollutant | Pearson r | Interpretation |
| --- | --- | --- |
| VOC | r = 0.125 | Weak positive correlation |
| PM 2.5 | r = -0.098 | Weak negative correlation |
| PM 10.0 | r = -0.059 | Weak negative correlation |

All three correlation values are very weak (|r| < 0.13), indicating that sensor altitude has little to no meaningful linear relationship with pollutant concentrations in this dataset. The slight positive correlation for VOC suggests that higher-elevation sensors in western Nebraska may record marginally higher VOC values, possibly reflecting the agricultural character of that region rather than altitude itself. The weak negative correlations for PM 2.5 and PM 10.0 suggest that lower-elevation sensors, which tend to be in eastern Nebraska near agricultural and industrial activity, show somewhat higher particulate concentrations on average. Overall, geographic location and proximity to emission sources appears to be a far stronger determinant of air quality than altitude alone in the Nebraska context.

# 4. Conclusion

This analysis of 8,301 daily observations from 27 AirPurple sensors across Nebraska from February 2024 through March 2025 provides a comprehensive characterization of air quality conditions across the state. Key findings include: Swnphd-Ogallala recorded the highest VOC concentrations (mean 399.434 ppb), while Broken Bow and Richardson County Courthouse showed extreme PM 10.0 values far exceeding NAAQS standards. A total of 25 PM 2.5 health risk event-days and 392 PM 10.0 health risk event-days were identified, with the most severe events reaching Hazardous AQI levels in January and February 2025.

Temperature and humidity were both found to influence pollutant concentrations in ways consistent with established atmospheric science. Very high humidity was associated with dramatically reduced concentrations across all pollutants, while cold temperatures were associated with elevated PM 10.0 from winter dust events. Two widespread PM 2.5 events on May 14 and July 25, 2024 affected multiple sensors simultaneously, indicating regional smoke transport events. Sensor altitude showed only a very weak correlation with pollutant concentrations, suggesting that geographic location and proximity to emission sources are more important drivers in Nebraska.

These findings provide the UNMC Water, Climate and Health Group with a data-driven basis for identifying locations and conditions that warrant targeted public health attention, continued monitoring, and potential community outreach efforts. Richardson County Courthouse and Broken Bow in particular warrant close monitoring given the frequency and severity of PM 10.0 events recorded during the study period.

# 5. References

AirNow. (2024). Air Quality Index (AQI) Basics. U.S. Environmental Protection Agency. Retrieved from https://www.airnow.gov/

Nebraska Department of Environment and Energy (NDEE). (2023). Nebraska Air Quality Report. State of Nebraska.

National Oceanic and Atmospheric Administration (NOAA). (2024). Wildfire Smoke and Air Quality. Retrieved from https://www.noaa.gov/

PurpleAir. (2024). PurpleAir Community Data Documentation. Retrieved from https://community.purpleair.com/c/data/api/18

U.S. Environmental Protection Agency. (2018). Technical Assistance Document for the Reporting of Daily Air Quality – the Air Quality Index (AQI). EPA-454/B-18-007.

U.S. Environmental Protection Agency. (2023). Criteria Air Pollutants: NAAQS Table. Retrieved from https://www.epa.gov/criteria-air-pollutants/naaqs-table